Drafting Basic Skirt Blocks:

Measurements:

Waist Circumference (plus .5” ease): ____________

Hip Circumference (plus 2” ease): _______________

Hip depth measurement at SS: _________________

Skirt Length (waist to knee) : ___________________

Skirt Block Draft Directions:

- Take your body measurements. Be sure to at ½” ease to your waist circumference and 2” ease to your hip circumference measurements and record those in the space provided above.

- Using your skirt length measurement draw a vertical line on the right side of your paper. Label the top point of this line  “A” and the bottom point  “B.

- Using half of your hip circumference measurement __________________ draw a horizontal line extending from point “B”. Label the end point of this line “C”.

- Complete the drafting rectangle by drawing a vertical line up from point “C” (label the end of this line point “D” and lastly draw a horizontal line from point “D” across to point “A”.

*** The rectangle should have a length that is equal to the skirt length measurement and a width that is equal to half of the waist circumference measurement.

- Label line  “AB” “Center Back or CB” and line “CD” “Center Front or CF”

- Divide the width of the rectangle in half  _________and use this measurement to measure from point A along line “AD” towards point “D”. Label this midpoint “E” and draw a vertical line down at this midpoint. Label the ending point of this line “F”.

- Use the hip depth measurement and measure down from point “A” along line “AB” and mark point  “G”. Draw a horizontal line from point “G” across and label the ending point “H”. Label line “GH” “Hipline”.

Calculate & Draft Darts:

- Calculate the difference between your waist circumference measurement and your hip circumference measurement ___________ (Your Dart intake amount for each dart plus  the amount that you subtract from the side seams should add up to this number).

***Note:  If the difference between the two measurements is less than 10” you can draft a single dart on the front and back of the skirt. If the difference is greater than 10” draft 2 darts on the front and back of the skirt for the best fit. ***

  - Calculations (for a single dart):

    - ½ hip circum. - ½ waist circum.= _________

    - Using the result from the above calculation subtract the back dart intake amount (1”-1 ¼ “ standard).

      - Difference between hip & waist circm - dart intake (1” or 1 ¼”) = _________

    - Using the result from the equation in “ii” subtract the front dart intake amount (½”-1” standard). The result of this equation is the left over amount that will need to be taken from the side seams.

    - Divide the left over amount in half and record result here: __________

    - Using the above recorded amount measure from point “E” to the left and mark point “I” then measure to the right of point “E” the same amount and mark point  “J”.

*** Points “I” and “J”  represent the side seam points along the waistline for the front and back skirt blocks***

- Measure the distance from point “A” to point “I” and find the midpoint _______. Make a notch to notate this midpoint. Then  divide the dart intake amount for your back dart in half _______ . Use this measurement to measure to the left and right of the midpoint (equal distance) and label your dart legs “K” and “L” respectively (“K” for left dart leg, “L” for right).

- At the midpoint notch of the dart, draw a vertical dashed line extending down from line “AD” towards line “GH”. Along this dashed line measure down 5”-5 ½” (from line“AD” toward line “GH”)and mark point “M”. Point “M” is your dart tip. Connect points “K” to “M” and “M” to “L” to form the back waist dart.

- Repeat the above steps to draft the front dart:

  - Find the midpoint of line “DJ” and mark that midpoint with a notch.

  - Divide the front dart intake amount _______.

  - Use this measurement to mark points to the left and right that are equal distance from the midpoint.

  - Label these points “N” (left side) and “O” (right side).

  - Draw a dashed line at the notched midpoint of the dart extending down from line “AD” toward line “GH”.

  - Measure down 3”-3 ½” (instead of the 5-5 ½” used for the back dart) and mark point “P”. ***The standard back dart depth amount is larger to compensate for the volume of the buttocks. ***

  - Connect point “N” to point “P” and point “P” to point “O” to form the front waist dart.

- Label the point where lines “AD” and “GH” meet point “Q”.

- At points “I” and “J” measure about ⅜” above each point and mark a dot .

- Use your hip curve to draw the side seams. Hip/side seam will extend from point “Q” through point “I” to the dot marked above point “I” to form back skirt side seam and from point “Q” through point “J” to the dot marked above “J” to form the front side hip curve/seam.

*** If your side hip curve is too deep (too curved) increase your dart intake amount or do a double dart instead of a single. If the hip curves are too shallow (too straight) decrease the dart intake amount.***

- Draw/ Blend your waist lines:

  - Extend each dart leg about ⅛ upward and the midpoint of each dart about ¼” upward. Connect those extended lines to create a small triangle for dart intake. ***you can also fold the darts closed to blend the waistline.